YEAAR 10 BASIC TECHNOLOGY

STRAND:HAND TOOLS AND APPLIANCES

Lesson 1: Geometrical Tools

Learning Outcome: Identify the tools & its uses

- Combination Square

- A combination square is a tool used for multiple purposes in woodworking.

- The combination square can be used for leveling,

- As  a try square, to determine the squareness of a piece of joint

- It can also be used as a saw guide.

2.	Sliding Bevel

- A sliding bevel is a tool which can be set to different angles to aid marking out.

- It is composed of two parts, the stock and the blade. The stock is usually made from rosewood which is a high quality material. The blade is made from hardened and tempered steel. It is used to mark lines at any angle on a work piece.

How to use a Sliding Bevel

To make a new angle

- Loosen the wing nut adjustment so that the blade is free to rotate.

- Swing the blade until the desired angle between the handle of the bevel and the blade side 	is made. This may be best achieved by placing a protractor on the inside of the bevel 	itself (so that the protractor is resting on the wing nut) and measuring out the desired 	angle.

- Lock the T bevel into place by tightening the wing nut.

- Lay the T bevel on your working surface and draw the angle by tracing along the inside 	of the T bevel blade and handle.

- Cut away.

To transfer an existing angle

- Loosen the wing nut adjustment so that the blade is free to rotate.

- Lay the T bevel against the surface of the angle cut to be copied.

- Rotate the inside edge of the blade to the existing angle until it matches the angle being 	transferred.

- Tighten the wing nut until it locks the blade and handle in position.

- Place the T bevel on the working surface and transfer the angle.

- Draw out the angle by tracing along the bevel on the inside edge of the blade and handle.

Setting the sliding Bevel to a slope of 1:6 for a d

ove tail joint

Lesson 2: Holding & Supporting tools

Learning Outcome: Understand the use of the tools & its parts

- Sash Cramps

Head

Slide

Bar

Thread

Sash clamps are used to clamp work together when it is glued.  They vary in size and are normally used in pairs.

- When in use, the sash clamp is placed below the work to  be glued / assembled.

- The slides are arranged on either side  and scrap wood is placed between each face and the work. This protects the work when the thread is tightened.

Lesson 3: Hand Saws

Learning Outcome: Identify the types of saws & its use

- Hand saws are hand-held tools, manually-driven, that are designed to cut through softer materials mainly wood.

- There are many different types of hand saws that vary based on how and what they cut.

(i)	Keyhole Saw

- A keyhole saw is perfect in cutting holes in wood and curves.

- It is used for cutting holes  in soft woods or in drywall, such as cutting a hole in a wall for a new  electric switch.

Blade

- Cross Cut Saw

- A cross cut saw has wide alternating bevel teeth perfect for rough  cutting on wood grains where tearing out is not important.

- It is used to cut large pieces of timber or cuts through a tree across the grain of timber.

- Panel Saw

- Panel saws are perfect for cutting small pieces of wood.

- It is especially used for cutting light boards like plywood across the  grain.

- Rip Cut Saw

- A rip cut saw is an aggressive, push stroke handsaw with sharpened  teeth top.

- It is specially designed for making cut parallel to the direction  of the wood grain.

- Back Saw

- Back saws are used for trimming and fine woodcutting.

Its teeth are smaller compared to other types of hand saws

- It is used for making fine accurate cuts in small pieces of wood such as cutting of joints, angles with and across the grain.

- Coping Saw

frame

blade

handle

Coping saws are perfect for cutting complex patterns on wood.

- A coping saw is a pull stroke hand saw.

- Hacksaw

- Hacksaws have fine, disposable blades held in  Tension by front and back pins.

- It is used to cut rods, bars, angle plates to 		required lengths   	and sheet metals to specific size and shape.

ACTIVITY

- Name the hand tool in the diagram given below and state the process being carried out.

- Name the part of the hand tool marked X given below

- Given: A sketch of a 45° set square on the right and corner A labelled.

Required: 	Redraw the set square to a scale of 1:1 starting from corner 				A given below.