WEEK 4				YEAR 11 APPLIED TECHNOLOGY

STRAND: ENGINEERING MATERIAL

LESSON 44: TYPES OF ABRASIVE BACKING

LEARNING OUTCOME: IDENTIFY THE TYPES OF ABRASIVE BACKING

- Type of abrasive backing

- Abrasive backing come in many materials depending on the intended application.

- Some of them are

i. Paper back – for general hand and machine sanding

ii. Cloth backing – for sanding belts used on sanding machines.

iii. Fibre backing – for machine sanding discs.

- On the abrasive back, letters used together with numbers represent the type and weight of the backing.

- Animal or resin glue or a combination of both is the common glue used in abrasive materials.

| Coarseness | Mesh or grit number | Application |
| --- | --- | --- |
| Very Coarse | 12,16,20,24,30 | Rough shaping forming of stock |
| Coarse | 36,40,50 | For removing deep marks and imperfection |
| Medium | 60,80,100 | Medium sanding before exterior painting or as an intermediate stage before fine sanding |
| Fine | 120,150,180 | Last sanding prior to the application of stain or paint |
| Very Fine | 220,240,280,320,360,400,500,600 | For rubbing and polishing finish after it has been applied and for cutting back surfaces between coats. |

LESSON 45: TYPES OF ABRASIVE

LEARNING OUTCOME: IDENTIFY THE TYPES OF ABRASIVE

- Aluminium Oxide Cloth Sand Sheets.

- Aluminium oxide cloth sheets are excellent for general clean up on metal and wood surfaces.

- They last longer than paper sheets.

- They have a flexible durable backing and are water resistant.

- Emery Cloth Sand Sheets

- Emery cloth sheets are close coat construction and are made of emery grain.

- They have a cloth backing which is very flexible.

- They are excellent for cleaning and polishing metal surfaces.

- They are also used on wood.

- Crocus Cloth Sand Sheets

- Crocus cloth sheets are made of a natural mineral used for extremely fine finishing and polishing on metals, plastics and woods.

- Silicon Carbide Waterproof Sand Paper

- Silicon carbide waterproof paper is best for wet and dry sanding of metal, paints and plastics.

- It leaves an excellent finish on most precious metals.

- It has a long life and is economical.

- Silicon Carbide Stearate Sand Paper

- Sheets Silicon carbide stearate coated paper sheets reduce loading of material and are excellent on wood, plastics and metals

- Garnet Finishing & Cabinet Sand Paper Sheets

- Garnet finishing & cabinet paper leaves an excellent finish on woods.

- It has a medium aggressiveness. It has a long life and is economical.

- Aluminium Oxide and Cabinet Sand Paper Sheets

- Aluminium oxide and cabinet paper is excellent for wood sanding.

- Sanding belts

- Aluminium oxide belts are resin bonded, and have tear and break resistant joints.

- They are excellent on all metals and hard woods however are also available for soft woods.

- Belts made of Aluminium oxide poly/cotton material have a specially coated backing which provides high performance for washable belts.

- Aluminium oxide premium cotton belts are flexible enough for most contour surfaces.

- Wheel dresser

- A Bench grinder wheel being dressed by a wheel dresser

- This removes the glazed surface of the grinding wheel, which will allow new cutting surface exposed.

LESSON 46: WOOD FINISHING

LEARNING OUTCOME: DEFINE WOOD FINISHING

- Wood Finishing

- Wood finishing is the process of applying to the surface after it has been prepared,

- by filling and smoothing

- a thin coating of varnish or other substance,

- to render it durable, enhance its beauty or change its appearance.

- There are numerous methods of finishing, and a variety of materials are used; the varieties of varnish being the principal.

- The distinctive qualities of these varieties are treated under their proper headings.

LESSON 47: VARNISH, PAINT, SEALERS

LEARNING OUTCOME: IDENTIFY VARNISH, PAINT & SEALERS

- Vanish

- Varnish is a transparent, hard, protective finish or film primarily used in wood finishing

- Varnish is traditionally a combination of a drying oil, a resin, and a thinner or solvent.

- Varnish finishes are usually glossy but may be designed to produce satin or semi-gloss sheens by the addition of "flatting" agents

- Varnish has little or no color, is transparent, and has no added pigment, as opposed to paints or wood stains, which contain pigment and generally range from opaque to translucent.

- Varnishes are also applied over wood stains as a final step to achieve a film for gloss and protection.

- Paint

- Paint is any liquid, liquefiable, or mastic composition that, after application to a substrate in a thin layer, converts to a solid film.

- It is most commonly used to protect, colour, or provide texture to objects.

- Paint can be made or purchased in many colours—and in many different types, such as water colour, artificial, etc.

- Paint is typically stored, sold, and applied as a liquid, but dries into a solid.

- Sealers

- Sealers are typically lower strength, yet flexible, bonding agents used between substrates of differing physical properties to form a seal between the materials.

- They seal top structures & particularly effective in waterproofing processes by keeping moisture out (or in) the components in which they are used.

- They can provide thermal and acoustical insulation, and may serve as fire barriers.

- They may have electrical properties

- Sealants can also be used for simple smoothing or filling.

LESSON 48: STAIN & GREASING/OILING

LEARNING OUTCOME: IDENTIFY STAIN & GREASING/OILING

- Stain

- A wood stain consists of a colorant suspended or dissolved in an agent or solvent.

- The suspension agent can be water, alcohol, petroleum distillate, or the actual finishing agent (shellac, lacquer, varnish, polyurethane, etc.)

- Coloured or 'stained' finishes, like polyurethane, do not penetrate the pores of the wood to any significant degree and will disappear when the finish itself deteriorates or is removed intentionally

- Greasing/Oiling

- Grease is a semisolid lubricant.

- It generally consists of a soap emulsified with mineral or vegetable oil.

- The characteristic feature of greases is that they possess a high initial viscosity

- Greases are applied to mechanisms that can only be lubricated infrequently and where a lubricating oil would not stay in position

- They also act as sealants to prevent ingress of water and incompressible materials.

- An oil is any neutral, nonpolar chemical substance that is a viscous liquid at ambient temperatures and is both hydrophobic (immiscible with water,  "water fearing") and lipophilic (miscible with other oils, literally "fat loving").

- Oils have a high carbon and hydrogen content and are usually flammable and slippery.

- Oils may be animal, vegetable, or petrochemical in origin, and may be volatile or non-volatile.

- They are used for food, fuel, lubrication, and the manufacture of paints, plastics, and other materials.

ACTIVITY

- Name the three types of abrasive backing

- Identify any three types of abrasive

- What is Wood Finishing?

- State the types of wood finishing

- Differentiate Varnish & Paint

- Mild steel is classified as a ferrous metal and copper as a non-ferrous metal.

- State the difference between ferrous and non-ferrous metals.

- Give a reason why non-ferrous metals have an advantage over ferrous metals

- List two characteristics of ceramic as a material.

- Concrete is a building material made by mixing together sand, crushed rock or gravel and cement.

- State the ratio of the ingredients used for a concrete mix.

- Give an advantage of using a mechanical concrete mixer over hand mixing of concrete.