YEAR 11 APPLIED TECHNOLOGY

WEEK 2

STRAND: ENGINEERING MATERIALS

LESSON 34: CONCRETE

LEARNING OUTCOME: IDENTIFY THE COMPONENTS OF CONCRETE

Cement

- Cement, the main ingredient of concrete, is a material in powder form which, when mixed with water, sets into a hard mass.

- The raw materials in the manufacture of cement are limestone (or chalk) and clay, shale and river mud.

Storing Cement

- When water is mixed with cement a chemical action takes place and the mixture will set into a hard mass.

- Even the moisture from the air or ground will cause this to happen, so the main condition in the storage of cement is to keep it dry.

- It should be stored dry on a raised platform and, if outside, covered with a tarpaulin.

Aggregates

- Aggregate consists of stone material such as rock or crushed gravel, which is added to the cement and water to form concrete or mortar.

- Aggregate ranges in size from the finest grains, ‘sand’, called ‘fine aggregate’, to the larger material called  ‘coarse aggregate’.

Water-Cement Ratio

- The proportion of water to cement, known as the water-cement ratio, is an important factor in the making of good concrete.

- If too much water is used the strength of the concrete will be lowered.

- The less water used the stronger will be the concrete. However, sufficient water is required to allow the mix to be workable.

LESSON 35: MIXING THE CONCRETE

LEARNING OUTCOME: ABLE TO MIX THE CORRECT MIX OF CONCRETE

Concrete mixing is carried out by hand or by mechanical batch mixers.

Proportions of Aggregates and Cement

- When the aggregates are proportioned by volume, it is the general practice to use one part of fine aggregate to two parts of coarse aggregate.

- The most commonly used mix by volume is 1:2:4—one measure of cement, two measures of sand, and four measures of coarse aggregate.

Mixing by Hand

- For hand mixing, the aggregates and sand are shoveled separately into the gauge box or measuring container and turned out on to a flat, watertight timber platform or flat, clean surface such as concrete.

Mixing by Machine

- A mechanical concrete mixing machine consists of a conical shaped drum in which blades are fixed.

- The drum is rotated on a framework to mix the materials.

Mechanical mixing is best carried out as follows:

- Add three-quarters of the water to the revolving drum.

- Add the correct quantities of coarse aggregate, cement and sand, in that order.

- Add the remaining water gradually until the mixture falls cleanly from the top of the mixer drum as it rotates.

LESSON 36: TESTING THE CONCRETE

LEARNING OUTCOME: UNDERSTAND THE IMPORTANCE OF TESTING CONCRETE

- The concrete slump test is an empirical test that measures the workability of fresh concrete.

- It measures the consistency of the concrete in that specific batch. This test is performed to check the consistency of freshly made concrete.

- concrete is tested to ensure that the material that was specified and bought is the same material that was delivered to you or the job site

LESSON 37: FIBERGLASS

LEARNING OUTCOME: IDENTIFY ITS COMPONENTS

- Fiberglass is a type of fiber reinforced plastic where the reinforcement fiber is specifically glass fiber.

- The glass fiber may be randomly arranged but is commonly woven into a mat.

- The glass fibers are made of various types of glass depending upon the fiberglass use.

- These glasses all contain silica or silicate, with varying amounts of oxides of calcium, magnesium, and sometimes boron.

LESSON 38: LEATHER

LEARNING OUTCOME: FIND OUT ITS USES & ORIGIN

- Leather is a durable and flexible material created by the tanning of animal rawhide and skin, often cattle hide.

- It can be produced through manufacturing processes ranging from cottage industry to heavy industry.

- Leather is used for various purposes including clothing (e.g. shoes, hats, jackets, skirts, trousers and belts), bookbinding, leather wallpaper, and as a furniture covering.

- Upholstery is the work of providing furniture, especially seats, with padding, springs, webbing, and fabric or leather covers.

ACTIVITY

- Name the cells of the softwoods and Hardwoods

- Give the purpose of the two types of barks:

- Outer bark –

- Inner bark –

- Give two differences apart from cell type between softwood and hardwoods.

- Give the purpose of the following parts of the tree:

- sapwood –

- Cambium layer –

- Pith –

- Heartwood