WEEK 7			YEAR 11 TECHNICAL DRAWING

STRAND: GEOMETRY

LESSON 31:  INTERPENETRATION-EQUAL CYCLINDER

LEARNING OUTCOME:  CONSTRUCT THE DEVELOPMENT OF CYCLINDER A

LESSON 32:  INTERPENETRATION-EQUAL CYCLINDER

LEARNING OUTCOME:  CONSTRUCT THE DEVELOPMENT OF CYCLINDER B

STRAND: APPLIED DRAWING

LESSON 33:  ORTHOGRAPHIC DRAWING

LEARNING OUTCOME:  DEFINE ORTHOGRAPHIC DRAWING

- Orthographic Projection is the representation of a three-dimensional (3D) component on a flat surface (the drawing sheet) in two-dimensional (2D) form.

- An orthographic drawing is also sometimes called a working drawing, is usually the last drawing produced by a designer.

- It normally has three accurate views of a product, a front view, side view and plan view.

- Dimensions (measurements) are also drawn on each view

LESSON 34:  ORTHOGRAPHIC DRAWING- BASIC RULES

LEARNING OUTCOME: IDENTIFY THE BASIC RULES OF ORTHOGRAPHIC DRAWING

- BASIC RULES OF ORTHOGRAPHIC DRAWING

| A plane surface that is perpendicular to a plane of projection appears on edge as a straight line | A plane surface that is parallel to a plane of projection appears as rectangle at true size | If a plane is angled to the plane of projection, it appears foreshortened 2 to 3 in projected plane is not equal to 2 to 3 measured on object |
| --- | --- | --- |

- WHEN IS SOMETHING “HIDDEN”

- When an edge is not visible from that view, it is shown as a hidden line as shown below.

LESSON 35:  ORTHOGRAPHIC DRAWING- FIRST ANGLE ORTHOGONAL

LEARNING OUTCOME: DRAW IN FIRST ANGLE ORTHOGRAPHIC DRAWING

- FIRST ANGLE ORTHOGRAPHIC DRAWING

- In first angle projection the object imagined to be in the first quadrant as shown on the right.

- The object is lies between the observer and plane of projection. The plane of projection is assumed to be non transparent.

- When views are drawn in their relative position; Top view comes below Front view, Right side view drawn to the left side elevation.

- A First Angle Projection drawing is identified by the first angle projection symbol. The angle of projection symbol typically appears in angle of projection block near to Title block.